# PCI-1752U PCI-1752USO

64-ch Sink (NPN) Isolated Digital Output

64-ch Source (PNP) Isolated Digital Output

2002/95/EC

RoHS

COMPLIANT

Features

- 64-ch Sink Type (NPN) Isolated Digital Output Module (PCI-1752U Only)

- 64-ch Source Type (PNP) Isolated Digital Output Module (PCI-1752USO Only)

- High-voltage isolation on output channels (2,500 VDC)

- Wide output range (5 ~ 40 VDC)

- High-sink/source current on isolated output channels (200 mA max./channel)

- Output status readback

- Keeps the output settings and values after system hot reset

- Channel-freeze function

- High-density 100-pin SCSI connector

Introduction

The PCI-1752USO is a 64-CH source type isolated digital output which provides a 2,500 V isolation protection. It features a wide output range from 5 to 40V, which makes it suitable for industrial automation applications.

Specifications

# Isolated Digital Output

- Channels	64 (16-ch/group)

- Output Type	PCI-1752U: Sink (NPN)

PCI-1752USO: Source (PNP)

- Isolation Protection	2,500 VDC

- Output Voltage	5 ~ 40 VDC

- Sink/ Source Current	200 mA max./channel

- Opto-isolator Response 100 μs

# General

- Bus Type Universal	PCI V2.2

- I/O Connectors	1 x 100-pin SCSI female connector

- Dimensions (L x H)	175 x 100mm (6.9" x 3.9")

- Power Consumption	Typical: 5 V @ 230 mA

Max.: 5 V @ 500 mA

- Operating Temperature 0 ~ 60°C (32 ~ 140°F)

- Storage Temperature	-20 ~ 70°C (-4 ~ 158°F)

- Storage Humidity	5 ~ 95%, RH non-condensing

Ordering Information

- PCI-1752U-BE	64ch Sink(NPN) Type Isolated Digital Output Card

- PCI-1752USO-BE	64ch Source(PNP) Type Isolated Digital Output Card

# Accessories

- PCL-10250-1E	100-pin SCSI to Two 50-pin SCSI Cable, 1 m

- PCL-10250-2E	100-pin SCSI to Two 50-pin SCSI Cable, 2 m

- ADAM-3951-BE	50-pin DIN-rail Wiring Board w/ LED Indicators